- Take out the batteries;

- Open “NuvoISP.exe”;

- Long press the “Fire Button” till the end of upgrading process;

- Connect your Kriemhild Box and your computer with the USB charging cable;

- Click on “connect”

- “Connected” in green color;

- Click on “APROM”

- Select this BIN file;

- Click “Open”;

- Select “APROM”, “Jump to APROM”, click “Start”;

- The Mod screen turns on, release the Fire button, unplug the USB cable;

- Long press the “+” “-” buttons at the same time, check the updated version “V2.6”.